CONTAINER LOADING TIPS

Parking the Container

When loading a container on a public road please check with local Police and your City if a permit is required. Also check for accessibility, the truck is over 14ft high and a 40ft container become 50ft plus, therefore no narrow roads, low trees and turning space for the driver is required.

Packing of The Goods

Proper packing and loading is your best insurance against damages. Here are a few tips to help you pack and load

Pack as much as you can into boxes. This will provide greater protection for your belongings and will make loading the container easier. Odd shaped items take up more space and make loading more difficult. Spending on packing supplies is are far cheaper than broken or damaged belongings

Blankets or sleeping bags you can buy from any thrift store used. These are to wrap around furniture, chairs, refrigerator, bed frames etc to prevent scratching

Moving Boxes you can buy from any U-haul Station or Moving Supplier, buy more boxes than you think you need as you can be refunded for any you do not use. You can get a variety of small, medium, and large, also dish packs, wardrobe boxes and even mattress boxes

Plastic sheeting from any paint supplier or Home Depot, wrap around any sofas, or large chairs etc

Pack several weeks before your moving date, taking it one room at a time. Begin with things you use less frequently, such as books etc. Pack each room in separate boxes

Reinforce the bottom of all boxes with plenty of tape. Fill each box to its capacity, using paper or fillers to fill any empty spaces. The top and sides of each box shouldn't bulge, they should not cave in when closed

Pack heavy items, such as books, in smaller boxes. Keep the weight of each box down to a manageable level, remember you will be picking this box up when the container arrives.

Leave lightweight linens and clothing in their drawers. Pack extra linens, pillows and stuffed animals in large garbage bags and tape closed. Plan to use these bags as cushions or fillers in your container. Disassemble any items you can. Be aware of items with sharp corners or projections and keep them from puncturing or scratching other objects.

Furniture

Furniture wrap with blankets or sleeping bags. Secure padding to furniture. Remove legs from furniture if possible. Place wing nuts or screws back on the unit they came from. Place sofa cushions and pillows in bags to use as pads or fillers in the container.

Move dressers with out drawers, remember the weight of the unit as you must be able to lift the unit up into the container, replace the drawers when your dresser is inside the container with contents, Secure drawers from opening during transit.

If you are moving a piano or other heavy unit, consider hiring a specialist to pack and load it

Electronics

Use original shipping boxes for computers, printers and other electronic items. Double boxing if required. Immobilize moving parts and remove all detachable cables and wires. Tape electrical cords to back of appliance to prevent plug damage. CDs and software do not survive high temperatures. If you are shipping your goods during the warmer seasons, consider keeping these items with you

Major Appliances

Clean and dry all appliances one day before you move. Remove broiler pan and racks from your oven. Tape burners or coil elements to the top of your stove and tape down all knobs. Disconnect and empty all washing machine hoses and store them inside the machine. Immobilize the tub with a brace or by placing towels between the tub and sidewall. Tape electrical cords to each appliance and tape all doors and lids closed

Dishes and Glassware

Wrap breakables in tissue paper and use newspaper to fill extra spaces in boxes. Pack plates on their edges. Consider using pot holders and dish towels to cushion the bottom and sides of your boxes. For greater protection, consider purchasing "dish packs" as previously mentioned

Fine silver should be wrapped in cloth or silver paper

Lamps and Decor

Remove all bulbs and shades from lamps. Pack in sturdy cartons labeled "fragile". Wrap each wall hanging individually with bubble wrap, blankets, or towels. Pack flat items on their edges. For large mirrors or pictures, cover both sides with bubble wrap and heavy cardboard, then bind with tape

Lawn and Garage Items

Drain all gasoline and oil from lawn mowers, chain saws and other equipment. Disconnect all batteries. Disconnect propane gas bottles from barbecue grill (you cannot ship the bottle). Strap long garden tools together into a bundle. Pack heavy power tools into small sturdy boxes and fill spaces with newspaper

Loading The Goods into the Container

Have all your goods boxed, wrapped and ready for loading into the container prior to containers arrival

The container stands 4ft above a chassis, so step ladders are a must.

Arrange plenty of help, 4 to 6 people loading a 20ft container would average out at 4 to 5 hours loading time, The more people the better to spread out the work, Remember to "squat" when handling heavier items and lift them with your knees, not your back. Ask for help with anything that tests your limits. Loading in a pre-planned order will greatly reduce loading time and fatigue

Number your boxes and make a brief note on the box contents. Boxes should have your name, address and tel #, (print labels from your PC) Mark fragile boxes for top loading only

Don't forget to make your packing list of the contents you put into the container

Materials and Equipment Check List :-

* 100ft to 200ft of Rope
* Knife
* Hammer
* Saw
* Wood 8 @ 4" x 2" x 8"
* Wood 2 @ 8" by 4" x 3/4' ply sheets
* Box of 3" Nails 
* Blankets
* Large Garbage Bags
* Moving Boxes
* Plastic sheeting*
* Tape
* Writing Pad *
* Black Markers
* 2 Bags of Cat Litter
* 1 or 2 Dolly's 
* 1 or 2 Step Ladders
* Padlock
* Preprinted Labels from a PC

Rope you would use every 4ft or in an X shape,(bottom left of container to top right, bottom right to top left) to hold the goods in place and prevent shifting

Plastic Sheeting is used over the top of the goods to catch any drips of water from condensation building up inside the container during transit

Cat Litter, you would simply cut the bag and leave open by the container door or on top of the plastic sheeting in order to help soak up any moisture from condensation

Load heavy boxes below and fragile boxes on top, large items can go in last which helps to shore up the smaller items. Load all items as tightly as possible to reduce shifting, rubbing and puncturing during transit. Be generous with blankets and padding, and place cardboard under anything that may be scuffed or soiled by riding directly on the floor of the container. Load mattresses and upholstered furniture up off of the floor. Remember to use bagged pillows, stuffed animals and furniture cushions as well as additional paper and cardboard to fill all gaps and spaces

Wood you would use to make a wooden bulkhead or wall after all goods are loaded in. I.E. you may load 18ft of a 20ft container so this bulkhead would hold the goods in place and stop any shifting

Last Steps

When the container is loaded do a walk though your home to double check nothings left over then the following last steps are required

* Put Padlock on container 
* Note container # (4 letters 7 numbers)
* Note Seal # (from driver)

What I cannot ship in my Boxes

### Items Your shipment should not include anything that is :-

### . Poisonous 
. Flammable 
. Corrosive
. Perishables
. Fuels / Oils & Paints / Varnishes / Insecticides / Liquid Bleach / Matches / Candles / Propane / Paint Thinners / Aerosol Cans / Cleaning Chemicals
. Food in Glass Jars
. Firearms or Ammunition
. Animals
. Drugs
. Jewelry
. Original Documents, I.E. Insurance Policy, Will, Birth Certificates
. Original Videos such as a Wedding Video
. Alcohol, Tobacco and Perfume Products
. Note "Australia and New Zealand requires clean goods, in particular they check for dirt on bike frames and tires, If wood is used for packing make sure there is no bark

### What if I have new stuff in there

### Any new products are likely subject to an import tax. Commercial Invoices will be required; this will not count as part of a house hold goods shipment seeking a tax free entry. This could cause an inspection of all the goods in the container from Customs therefore we strongly recommend you ship any new product or goods separately